# Functional requirement specification AFT me-zkl3000switch-board

# Revision History

| Rev. nr. | Description | Date | Status | Author |
| --- | --- | --- | --- | --- |
| 0 | First concept, definition of functional requirements and derivation of required test pads. Detailed description of each requirement still missing. | 2017-03-06 | Draft | GM |
| 0.1 | Adding TST-FRS13: test to measure the quality of the short. Also added relevant test locations on rail connectors | 2017-03-07 | Draft | GM |
| 1.0 | Reviewed by Dual Inventive | 2017-03-08 | Accepted | GM |

| Initials | Name | E-mail address |
| --- | --- | --- |
| GM | Gijs Meuleman | g.meuleman@meuleman.io |

# Scope

This document specifies requirements for the automatic factory test (AFT) board for the me-zkl3000switch-board. An automated factory test board is used to verify if a board is produced correctly. It aims to test that all components are assembled correctly and that each component functions as expected.

The automatic factory test board is limited to testing single boards only. It does not test if a set of boards correctly works together. That is part of an integration test, which is beyond the scope of this document.

# Purpose

The purpose of this document is to record which capabilities and constraints are applicable for the AFT-board of the me-zkl3000switch-board.

# List of abbreviations

## Requirement categories

| ID | Requirement description |
| --- | --- |
| GEN | General requirements |
| TST | Test related requirements |

## Project / product abbreviations

| ABBR | Description |
| --- | --- |
| AFT | Automatic Factory Test |
| ZKL | Zelf Kortsluit Lans |
| MCU | Microcontroller unit |

# List of requirements

| Requirement ID. | Description |
| --- | --- |
| GEN-FRS-01 | A version management system needs to be constructed to register the changes made in the product. |
| GEN-FRS-02 | It should connect to the me-zkl3000switch-board using test pins. |
| GEN-FRS-03 | It should connect to a Raspberry Pi over USB |
| GEN-FRS-04 | It should be possible to use the board when directly connected to a PC over USB, e.g. it should be possible to bypass the Raspberry Pi. |
| GEN-FRS-05 | It should be easy insert and remove the me-zkl3000switch-board |
| GEN-FRS-06 | Using a simple user interface it should be possible to start the test and visualize the result (pass/fail). |
| GEN-FRS-07 | A log file of the test will be sent to a remote server. |
| TST-FRS-01 | It should test if the following power supply voltages are in range: +3V3, +3V3-MCU, +5V |
| TST-FRS-02 | It should test if the +3V3-MCU can be switched on/off |
| TST-FRS-03 | For each microcontroller, it should be able to program the MCU |
| TST-FRS-04 | For each microcontroller, it should command the MCU to go to the desired test mode using UART |
| TST-FRS-05 | For each microcontroller, it should test if the versioning reads 1,0,0,0 |
| TST-FRS-06 | For each microcontroller, it should test if the MCU can send/receive data using CAN |
| TST-FRS-07 | For each Switch Power Section, it should test the position of the switch in on/off mode |
| TST-FRS-08 | For each Switch Power Section, it should test the battery-low indicator during a low voltage (6.5V) and high voltage (7V). |
| TST-FRS-09 | It should test the inputs and outputs of the Key Switch Connector |
| TST-FRS-10 | It should power the board using the Main Connector. |
| TST-FRS-11 | It should test the CAN bus interface of the Main Connector. |
| TST-FRS-12 | It should test the power supply and the CAN bus interface of the CAN Extension Connector. |
| TST-FRS-13 | It should be possible to test the quality of the short (in milliohms) of each series section. This should be done |

## General requirements

| GEN-FRS-01 | A version management system needs to be constructed to register the changes made in the product |
| --- | --- |
| Priority | Must have |
| Status |  |
| Reference |  |
| Rationale | For feasibility testing of the new product, it is important to construct a version management system to document the changes made in xx (e.g. design, (embedded) software, electronics) and to register the impact it has on the usability of the device. |
| Comments |  |

To be expanded

## Test related requirements

| TST-FRS-01 | It should test if the following power supply voltages are in range: +3V3, +3V3-MCU, +5V |
| --- | --- |
| Priority | Must have |
| Status |  |
| Reference |  |
| Rationale | This allows to verify the correct voltages on the board and checks if there are no errors made during assembly of e.g. voltage divider resistors. |
| Comments |  |

To be expanded

# List of test pins

Based on the functional requirements, the following list of test pins can be derived.

Sheet: TopLevelSheet

No test pads needed here.

Sheet: SingleSwitchSection-[n], where n is from 1 to 4;

| Function | Net name | Comment |
| --- | --- | --- |
| BATP | J10-2, J11-2, J14-2, J15-2 | THT component, no pad needed |
| BATM | J10-1, J11-1, J14-2, J15-1 | THT component, no pad needed |

Sheet: SwitchPower

No test pads needed here.

Sheet: SineWaveGenerator

No test pads needed here.

Sheet: [s]MicroController, where [s] can be Measurement, Drive, or Control

| Function | Net name | Comment |
| --- | --- | --- |
| MCU-X-SWDIO | J5-2, J7-2, J9-2 | THT component, no pad needed |
| GND | J5-3, J7-3, J9-3 | THT component, no pad needed |
| MCU-X-SWDCLK | J5-4, J7-4, J9-4 | THT component, no pad needed |
| MCU-X-SWO | J5-6, J7-6, J9-6 | THT component, no pad needed |
| MCU-X-RESET-N | J5-10, J7-10, J9-10 | THT component, no pad needed |
| GND | J4-1, J6-1, J8-1 | Pad required, connector not assembled in production |
| UART-RX | J4-4, J6-4, J8-4 | Pad required, connector not assembled in production |
| UART-TX | J4-5, J6-5, J8-5 | Pad required, connector not assembled in production |

Sheet: PowerSupplyUnit

| Function | Net name | Comment |
| --- | --- | --- |
| +3V3 | +3V3 |  |
| +3V3-MCU | +3V3-MCU |  |
| +5V | +5V |  |
| GND | GND | Close to power supply IC U8 |

Sheet: Connectors

| Function | Net name | Comment |
| --- | --- | --- |
| INPUT-1 | J1-2 | Separate test cable, no pad needed |
| INPUT-2 | J1-4 | Separate test cable, no pad needed |
| DETECTION-LED | J1-5 | Separate test cable, no pad needed |
| +VBAT | J2-1 | Separate test cable, no pad needed |
| GND | J2-3 | Separate test cable, no pad needed |
| CANH | J2-5 | Separate test cable, no pad needed |
| CANL | J2-6 | Separate test cable, no pad needed |
| +VBAT | J3-1 | Separate test cable, no pad needed |
| GND | J3-3 | Separate test cable, no pad needed |
| CANH | J3-5 | Separate test cable, no pad needed |
| CANL | J3-6 | Separate test cable, no pad needed |

Sheet: RailConnectors

| Function | Net name | Comment |
| --- | --- | --- |
| Rail-A1 | Rail-A | THT component, no pad needed; Multiple pads required for higher current capabilities |
| Rail-A2 | Rail-A | THT component, no pad needed; Multiple pads required for higher current capabilities |
| Rail-A3 | Rail-A | THT component, no pad needed; Test location needed to measure voltage |
| Rail-B1 | Rail-B | THT component, no pad needed; Multiple pads required for higher current capabilities |
| Rail-B2 | Rail-B | THT component, no pad needed; Multiple pads required for higher current capabilities |
| Rail-B3 | Rail-B | THT component, no pad needed; Test location needed to measure voltage |