# Board review me-zkl3000switch-board

# Revision History

| Rev. nr. | Description | Date | Status | Author |
| --- | --- | --- | --- | --- |
| 0.1 | First concept |  | Draft | GM |
| 0.2 | Update functional requirements, update guideline checklist | 13-03-17 | Draft | GM |

| Initials | Name | E-mail address |
| --- | --- | --- |
| GM | Gijs Meuleman | g.meuleman@meuleman.io |

# Product requirements

The table below shows the product requirements for the schematics.

| ID. | Description | Checked |
| --- | --- | --- |
| SB-1 | Switch power sections and glue-logic must be retained except for the emergency switch circuitry: - Original design is available in PDF - Most is already redrawn in Altium Designer - The power MOSFET’s must be placed on the bottom, these are clamped against the back-plate with thermal conductive rubber for heat dissipation - Mounting hole spacers (m3) necessary for mounting to heatsink - NOTE: The switch circuit now operates on 5V but the new microcontrollers only work at 3.3V. Probably the switch circuit must be converted to work at 3.3V because the minimum battery voltage is 5V. | OK |
| SB-2 | 3x Microcontroller STM32F1 series o With a 8MHz/30ppm crystal oscillator clock source (farnell 2101329) | OK |
| SB-3 | 3x Internal SWD connector (one for each microcontroller) - 5x2-pin 1.27mm male header - Pinout: - 1: +3V3 (MCU-voltage) - 2: SWDIO - 3: GND - 4: SWCLK - 5: GND - 6: SWO - 7: NC - 8: NC - 9: GND - 10: RESET_n | OK |
| SB-4 | 3x Internal UART connector (one for each microcontroller) (when possible) - 6x1-pin 2.54mm male header - Pinout: - 1: GND - 2: NC - 3: NC - 4: MCU_RX - 5: MCU_TX - 6: NC | OK |
| SB-5 | 3x CAN Transceiver (one for each microcontroller) o SN65HVD230D | OK |
| SB-6 | The battery board (connected to the switch board) is unchanged - This board is a board with CR2032 batteries which is exchanged multiple times in the lifetime of the Switch board. - Connector placement and board outline are available - Connectors: - 4x metal stand-offs - Wurth 9774080360 - 4x 2px2 2.54mm male headers - There is a 5th 2px2 2.54mm male header, but this may be left not connected | OK |
| SB-7 | CAN, power connector see Section 3.5 | OK |
| SB-8 | Key-switch Connector - Wurth 690367290676 - Pinout: - 1: GND - 2: Input 1 - 3: GND - 4: Input 2 - 5: Detection LED anode signal with 10 ohm series resistor (for ZKL3000 RC-C hand switch) - 6: NC - The inputs are directly connected to the inputs of the ‘Control microcontroller’ | OK |
| SB-9 | Rail connector A & B o Wurth 7460719 | OK |
| SB-10 | CAN-extension connector (e.g CAN-controlled LED-bar). The power is unregulated to compensate for voltage losses in cabling (same pinout as the interconnect between Main Board and Switch Board see section 3.5). - Wurth 690367290676 - Pinout: - 1:V+ - 2: V+ - 3: GND - 4: GND - 5: CANH - 6: CANL - 7: GND - 8: GND - 9: V+ - 10: V+ | OK |
| SB-11 | Manual test points (metalized) must be easily accessible for trouble shooting (top side) - Silkscreen annotations of test points (when possible) - Important signals (TBD) | OK |
| SB-12 | With regards to the previous Switch board the following components need a parallel 100nF capacitor: - R46 - R83 - R120 - R157 | OK |

# FMEA

n/a

# Guidelines checklist

Schematics

| # | Description | Comments | Checked |
| --- | --- | --- | --- |
| GS-1 | Project compiles successfully |  | OK |
| GS-2 | Price and availability checked for each component |  | OK |
| GS-3 | Are there sufficient decoupling capacitors |  | OK |

PCB

| # | Description | Comments | Checked |
| --- | --- | --- | --- |
| GP-1 | Footprint matches component layout (check with paper print) |  | OK |
| GP-2 | There is at least a project name, version and date on the PCB |  | OK |
| GP-3 | Component that produce heat should be placed on the top of the board |  | OK |
| GP-4 | Use through hole connectors because they are stronger, wherever possible |  | OK |
| GP-5 | Decoupling capacitors close to required pins |  | OK |
| GP-6 | Draw width of mating connectors on the silkscreen to make sure that they fit |  | OK |
| GP-7 | Place testpoints to accommodate testbed. Give it testpoints an informative label |  | OK |
| GP-8 | Place fiducials on top and bottom. On large PCB's place them also close to high-density IC's |  | OK |
| GP-9 | Are screws not touching copper on top layer? |  | OK |
| GP-10 | Are there mounting holes on PCB? |  | OK |
| GP-11 | Are all configuration components (jumpers,buttons, configuration resistors etc) accessible? |  | OK |
| GP-12 | In module/baseboard configuration, is pin 1 connector equal to other pin 1 connector? |  | OK |
| GP-13 | Does every component have an assembly drawing outline and designator? |  | OK |

Gerber files

| # | Description | Checked |
| --- | --- | --- |
| GC-1 | Check top/bottom layers and corresponding solder mask. Does the solder mask not expose two neighboring tracks? Rationale: Neighboring tracks could get shorted when solder paste is applied, resulting in board failure. | OK |
| GC-2 | Check if all through hole via’s are closed (i.e. have solder mask applied) Rationale: If via’s are open, the via’s could start to oxidate, something that should be avoided. Although it is handy during prototype phase (you can directly apply a probe), it is not recommended for production. | OK |
| GC-3 | Check if power planes have no solder necks. Rationale: Obvious, resulting in too much resistance | OK |
| GC-4 | Check if high-speed signals have a power or ground plane below them for best signal integraty Rationale: Better signal integrity | OK |
| GC-5 | Check parallel tracks on different layers, minimize it Rationale: Long parallel tracks result in cross talk. This is especially dangerous for the combination of a high-speed signal and a reset signal! | OK |
| GC-6 | Check each word on the PCB, read it out lout Rationale: You don’t want spell errors on PCB’s that are manufactured in large quantities. | OK |
| GC-7 | Check the specification file. Read it carefully Rationale: Provides information on how the board should be fabricated. Has to be correct. | OK |