Board review me-zkl3000-switch-board

# General information

| Project | PME1044 Dual Inventive Switch Design |  |
| --- | --- | --- |
| Versioning | Refer to changes at end of document |  |
| Client | Dual Inventive |  |
| Client’s Project | 71027 ZKL 3000 RC 2.0 |  |
| Reviewers | Gijs Meuleman, Jerry Jacobs, San Bergmans |  |
| Block diagram | See document ‘product requirements’ |  |
| Reviewed schematic | 2017-xx-xx me-zkl3000-switch-board.pdf | Version: - |
| Product requirements | 71027 Hardware Design requirements v1.01.pdf | Version: 1.01 13-12-16 |
| FMEA | n/a |  |

# Schematics

## Product requirements

| ID. | Description | Checked |
| --- | --- | --- |
| SB-1 | Switch power sections and glue-logic must be retained except for the emergency switch circuitry: - Original design is available in PDF - Most is already redrawn in Altium Designer - The power MOSFET’s must be placed on the bottom, these are clamped against the back-plate with thermal conductive rubber for heat dissipation - Mounting hole spacers (m3) necessary for mounting to heatsink - NOTE: The switch circuit now operates on 5V but the new microcontrollers only work at 3.3V. Probably the switch circuit must be converted to work at 3.3V because the minimum battery voltage is 5V. |  |
| SB-2 | 3x Microcontroller STM32F1 series o With a 8MHz/30ppm crystal oscillator clock source (farnell 2101329) |  |
| SB-3 | 3x Internal SWD connector (one for each microcontroller) - 5x2-pin 1.27mm male header - Pinout: - 1: +3V3 (MCU-voltage) - 2: SWDIO - 3: GND - 4: SWCLK - 5: GND - 6: SWO - 7: NC - 8: NC - 9: GND - 10: RESET_n |  |
| SB-4 | 3x Internal UART connector (one for each microcontroller) (when possible) - 6x1-pin 2.54mm male header - Pinout: - 1: GND - 2: NC - 3: NC - 4: MCU_RX - 5: MCU_TX - 6: NC |  |
| SB-5 | 3x CAN Transceiver (one for each microcontroller) o SN65HVD230D |  |
| SB-6 | The battery board (connected to the switch board) is unchanged - This board is a board with CR2032 batteries which is exchanged multiple times in the lifetime of the Switch board. - Connector placement and board outline are available - Connectors: - 4x metal stand-offs - Wurth 9774080360 - 4x 2px2 2.54mm male headers - There is a 5th 2px2 2.54mm male header, but this may be left not connected |  |
| SB-7 | CAN, power connector see Section 3.5 |  |
| SB-8 | Key-switch Connector - Wurth 690367290676 - Pinout: - 1: GND - 2: Input 1 - 3: GND - 4: Input 2 - 5: Detection LED anode signal with 10 ohm series resistor (for ZKL3000 RC-C hand switch) - 6: NC - The inputs are directly connected to the inputs of the ‘Control microcontroller’ |  |
| SB-9 | Rail connector A & B o Wurth 7460719 |  |
| SB-10 | CAN-extension connector (e.g CAN-controlled LED-bar). The power is unregulated to compensate for voltage losses in cabling (same pinout as the interconnect between Main Board and Switch Board see section 3.5). - Wurth 690367290676 - Pinout: - 1:V+ - 2: V+ - 3: GND - 4: GND - 5: CANH - 6: CANL - 7: GND - 8: GND - 9: V+ - 10: V+ |  |
| SB-11 | Manual test points (metalized) must be easily accessible for trouble shooting (top side) - Silkscreen annotations of test points (when possible) - Important signals (TBD) |  |
| SB-12 | With regards to the previous Switch board the following components need a parallel 100nF capacitor: - R46 - R83 - R120 - R157 |  |

## Guidelines

| # | Description | Checked |
| --- | --- | --- |
| GL-1 | Project compiles successfully |  |
| GL-1 | Price and availability checked for each component |  |
| GL-3 | Are there sufficient decoupling capacitors |  |

## Corrections

| Problem Description | Corrective action | Checked |
| --- | --- | --- |

# FMEA

n/a

# PCB

## Guidelines

| # | Description | Comments | Checked |
| --- | --- | --- | --- |
|  | Footprint matches component layout (check with paper print) |  |  |
|  | There is at least a project name, version and date on the PCB |  |  |
|  | Component that produce heat should be placed on the top of the board |  |  |
|  | Use through hole connectors because they are stronger, wherever possible |  |  |
|  | Decoupling capacitors close to required pins |  |  |
|  | Draw width of mating connectors on the silkscreen to make sure that they fit |  |  |
|  | Place testpoints to accommodate testbed. Give it testpoints an informative label |  |  |
|  | Place fiducials on top and bottom. On large PCB's place them also close to high-density IC's |  |  |
|  | Are screws not touching copper on top layer? |  |  |
|  | Are there mounting holes on PCB? |  |  |
|  | Are all configuration components (jumpers,buttons, configuration resistors etc) accessible? |  |  |
|  | In module/baseboard configuration, is pin 1 connector equal to other pin 1 connector? |  |  |
|  | Does every component have an assembly drawing outline and designator? |  |  |

## Corrections

| Problem Description | Corrective action | Checked |
| --- | --- | --- |

# Document Revisions

| Revision | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | 31-1-2017 | GM, DY | First draft. |

# Document Authors

| Initials | Name | E-mail address |
| --- | --- | --- |
| GM | Gijs Meuleman | g.meuleman@meuleman.io |
| DY | David Young | d.young@meuleman.io |

[End-of-document]