# Design Document

# me-zkl3000-switch-board

# Sheet: SwitchPower

| Key Design Choices | Reason |
| --- | --- |
| SwitchPower Circuit | Previous version retained/ Same functionality |

# Sheet: SingleSwitchSection-x

| Key Design Choices | Reason |
| --- | --- |
| SingleSwitchSection | Most of the previous version retained/ Same functionality. Circuit related to emergency signals removed as these signals are obsolete in new version. +5V to +3V3 level shifting circuit is added for RDS-LOW and BATT-LOW signals to suit voltage level of new microcontroller |

# Sheet: SineWaveGenerator

| Key Design Choices | Reason |
| --- | --- |
| SinWaveGenerator Circuit | Previous version retained/ Same functionality |

# Sheet: MeasurementMicrocontroller/ DriveMicrocontroller/ ControlMicrocontroller

| Key Design Choices | Reason |
| --- | --- |
| Microcontroller: STM32F103CBT6/LQFP48 | As per specs: STM32F1 series Preferably low pin count Used in TWS designs |
| CAN Transceiver: SN65HVD230D/SOIC8 | Mentioned in requirements |
| Pin assignment for microcontroller | According to the signal functionalities |
| Crystal and corresponding circuit | Crystal mentioned in requirements Load Capacitance value according to datasheet |
| CAN Resistors | According to value in datasheets 0402 power rating is fine for normal operating conditions |
| Connectors and Versioning | As per specifications and review inputs 17-01-17 |
| Power Section | Derived from TWS-DUU design and datasheet |

# Sheet: PowerSupplyUnit

| Key Design Choices | Reason |
| --- | --- |
| +3V3 Switching Power Supply and Corresponding Circuit | Input voltage range, output voltage (adjustable) and output current requirements Chosen from the same series of SPS used in TWS-DUU Circuit derived from datasheet. Resistor divider formula: Vout = 3.3V, Vref = 0.8 V R1 = 7.5K, R2 = 2.4K |
| +5V Regulator and Corresponding Circuit | Previous version retained/ Same functionality |
| +3V3-MCU circuit | Based on review inputs 21-12-16 |

# Sheet: Connectors

| Key Design Choices | Reason |
| --- | --- |
| Connector circuit | Based on specs and review inputs 21-12-16 |

# Sheet: RailConnectors

| Key Design Choices | Reason |
| --- | --- |
| RailConnectors | Based on specs |