# Hardware Test Results me-zkl3000-switch-board

Based on schematics: 	2017-01-27 me-zklmain-board.pdf

# Power Supply Unit +12V

Prerequisites: Connect 12V power supply to +VBAT

| Test ID | HT-01 |
| --- | --- |
| Test procedure | Check +VBAT for +12V |
| Expected result | 12V |
| Measured | Voltage at Main Board = +12.02 V Voltage at Switch Board = +11.75 V |

| Test ID | HT-02 |
| --- | --- |
| Test procedure | Check +3V3 regulator output with respect to +VBAT (12V) |
| Expected result | Output at 3.3V |
| Measured | Output at +3.298 V |

| Test ID | HT-03 |
| --- | --- |
| Test procedure | Check +5V regulator output with respect to +VBAT (12V) |
| Expected result | Output at +5V |
| Measured | Output at +4.997 |

| Test ID | HT-04 |
| --- | --- |
| Test procedure | Check +3V3-MCU with +VBAT at 12V - Make MCU-SUPPLY-EN high - Make MCU-SUPPLY-EN low |
| Expected result | - Output = +3.3V by default and when MCU-SUPPLY-EN is high - Output = 0V when MCU-SUPPLY-EN is low |
| Measured | - Output = +3.294 by default and when MCU-SUPPLY-EN is high - Output = +0.924 V when MCU-SUPPLY-EN is low |
| Comments | A pull down at +3V3-MCU will enable the line to go completely to zero |

# Power Supply Unit +6.4V

Prerequisites: Connect 6.4V power supply to +VBAT

| Test ID | HT-05 |
| --- | --- |
| Test procedure | Check +VBAT for +6.4V |
| Expected result | 6.4V |
| Measured | Input Voltage at Main Board = 6.45 V Voltage at Switch Board = 6.15 V |

| Test ID | HT-06 |
| --- | --- |
| Test procedure | Check +3V3 regulator output with respect to +VBAT (6.4V) |
| Expected result | Output at 3.3V |
| Measured | Output at +3.298 V |

| Test ID | HT-07 |
| --- | --- |
| Test procedure | Check +5V regulator output with respect to +VBAT (6.4V) |
| Expected result | Output at +5V |
| Measured | Output at +4.996 V |

| Test ID | HT-08 |
| --- | --- |
| Test procedure | Check +3V3-MCU with +VBAT at 6.4V - Make MCU-SUPPLY-EN high - Make MCU-SUPPLY-EN low |
| Expected result | - + Output = +3.3V by default and when MCU-SUPPLY-EN is high - Output = 0V when MCU-SUPPLY-EN is low |
| Measured | - Output = +3.295 by default and when MCU-SUPPLY-EN is high - Output = +0.926 V when MCU-SUPPLY-EN is low |
| Comments | A pull down at +3V3-MCU will enable the line to go completely to zero |

# Sine Wave Generator

Prerequisites: EN-SINEGEN is high, +5V Power Supply is active

| Test ID | HT-09 |
| --- | --- |
| Test procedure | Check sine wave generator |
| Expected result | Sine wave of 118KHz with Vpp of 4V at output of U9B (green signal in the picture below) |
| Measured | Sine wave of 116KHz with Vpp of about 4V at output of U9B |

| Test ID | HT-10 |
| --- | --- |
| Test procedure | Check comparator threshold signal |
| Expected result | A smoothed out rectified wave of about 600mV at C14 (Yellow signal in picture below) |
| Measured | A smoothed out rectified wave of about 650mV at C14 |

# Control Microcontroller

Prerequisites: +3V3 Power Supply is active

| Test ID | HT-11 |
| --- | --- |
| Test procedure | Connect programming cable Program device |
| Expected result | OK |
| Measured | OK |

| Test ID | HT-12 |
| --- | --- |
| Test procedure | Check debug UART |
| Expected result | should be printing UART characters on PC terminal |
| Measured | Printing and Receiving UART characters through PC terminal |

| Test ID | HT-13 |
| --- | --- |
| Test procedure | Check versioning - Read VER0-VER3 |
| Expected result | Should read 1,0,0,0 |
| Measured | VER0 – VER3 = 1,0,0,0 |

| Test ID | HT-14 |
| --- | --- |
| Test procedure | Check CAN - Transmit message at 500 KHz bit rate - Receive message at Measurement and Drive Microcontroller - Transmit message from Measurement and Drive Microcontroller - Receive message at Control Microcontroller |
| Expected result | Transmitted and received message should be equal |
| Measured | Transmit and Receive messages are equal |

# Measurement Microcontroller

Prerequisites: +3V3-MCU Power Supply is active

| Test ID | HT-15 |
| --- | --- |
| Test procedure | Connect programming cable Program device |
| Expected result | OK |
| Measured | OK |

| Test ID | HT-16 |
| --- | --- |
| Test procedure | Check debug UART |
| Expected result | should be printing UART characters on PC terminal |
| Measured | Printing and Receiving UART characters through PC terminal |

| Test ID | HT-17 |
| --- | --- |
| Test procedure | Check versioning - Read VER0-VER3 |
| Expected result | Should read 1,0,0,0 |
| Measured | VER0 – VER3 = 1,0,0,0 |

| Test ID | HT-18 |
| --- | --- |
| Test procedure | Check CAN - Transmit message at 500 KHz bit rate - Receive message at Control and Drive Microcontroller - Transmit message from Control and Drive Microcontroller - Receive message at Measurement Microcontroller |
| Expected result | Transmitted and received message should be equal |
| Measured | Transmit and Receive messages are equal |

# Drive Microcontroller

Prerequisites: +3V3-MCU Power Supply is active

| Test ID | HT-19 |
| --- | --- |
| Test procedure | Connect programming cable Program device |
| Expected result | OK |
| Measured | OK |

| Test ID | HT-20 |
| --- | --- |
| Test procedure | Check debug UART |
| Expected result | should be printing UART characters on PC terminal |
| Measured | Printing and Receiving UART characters through PC terminal |

| Test ID | HT-21 |
| --- | --- |
| Test procedure | Check versioning - Read VER0-VER3 |
| Expected result | Should read 1,0,0,0 |
| Measured | VER0 – VER3 = 1,0,0,0 |

| Test ID | HT-22 |
| --- | --- |
| Test procedure | Check CAN - Transmit message at 500 KHz bit rate - Receive message at Control and Measurement Microcontroller - Transmit message from Control and Measurement Microcontroller - Receive message at Drive Microcontroller |
| Expected result | Transmitted and received message should be equal |
| Measured | Transmit and Receive messages are equal |

# Single Switch Section-1

Prerequisites: All power supplies are active, Battery Board or suitable supply is connected

| Test ID | HT-23 |
| --- | --- |
| Test procedure | Check Switch Section 1 ON Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R79 (SECTION1-ON) - Ensure DC voltage at R81 (SECTION1-OFF) |
| Expected result | Signal at R70 should be high (RDS-LOW-1) |
| Measured | RDS-LOW-1 is logic high |

| Test ID | HT-24 |
| --- | --- |
| Test procedure | Check Switch Section 1 OFF Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R81 (SECTION1-OFF) - Ensure DC voltage at R79 (SECTION1-ON) |
| Expected result | Signal at R70 should be low (RDS-LOW-1) |
| Measured | RDS-LOW-1 is logic low |

| Test ID | HT-25 |
| --- | --- |
| Test procedure | Check Switch Section 1 Battery High Function - Ensure voltage above +7V across J10 connector |
| Expected result | Signal at R76 should be high (BAT-LOW-1) |
| Measured | BAT-LOW-1 is logic high |

| Test ID | HT-26 |
| --- | --- |
| Test procedure | Check Switch Section 1 Battery Low Function - Ensure voltage below +6.5V across J10 connector |
| Expected result | Signal at R76 should be low (BAT-LOW-1) |
| Measured | BAT-LOW-1 is logic low |

# Single Switch Section-2

Prerequisites: All power supplies are active, Battery Board or suitable supply is connected

| Test ID | HT-27 |
| --- | --- |
| Test procedure | Check Switch Section 2 ON Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R96 (SECTION2-ON) - Ensure DC voltage at R98 (SECTION2-OFF) |
| Expected result | Signal at R87 should be high (RDS-LOW-2) |
| Measured | RDS-LOW-2 is logic high |

| Test ID | HT-28 |
| --- | --- |
| Test procedure | Check Switch Section 2 OFF Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R98 (SECTION2-OFF) - Ensure DC voltage at R96 (SECTION2-ON) |
| Expected result | Signal at R87 should be low (RDS-LOW-2) |
| Measured | RDS-LOW-2 is logic low |

| Test ID | HT-29 |
| --- | --- |
| Test procedure | Check Switch Section 2 Battery High Function - Ensure voltage above +7V across J11 connector |
| Expected result | Signal at R93 should be high (BAT-LOW-2) |
| Measured | BAT-LOW-2 is logic high |

| Test ID | HT-30 |
| --- | --- |
| Test procedure | Check Switch Section 2 Battery Low Function - Ensure voltage below +6.5V across J11 connector |
| Expected result | Signal at R93 should be low (BAT-LOW-2) |
| Measured | BAT-LOW-2 is logic low |

# Single Switch Section-3

Prerequisites: All power supplies are active, Battery Board or suitable supply is connected

| Test ID | HT-31 |
| --- | --- |
| Test procedure | Check Switch Section 3 ON Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R133 (SECTION3-ON) - Ensure DC voltage at R135 (SECTION3-OFF) |
| Expected result | Signal at R124 should be high (RDS-LOW-3) |
| Measured | RDS-LOW-3 is logic high |

| Test ID | HT-32 |
| --- | --- |
| Test procedure | Check Switch Section 3 OFF Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R135 (SECTION1-OFF) - Ensure DC voltage at R133 (SECTION1-ON) |
| Expected result | Signal at R124 should be low (RDS-LOW-3) |
| Measured | RDS-LOW-3 is logic low |

| Test ID | HT-33 |
| --- | --- |
| Test procedure | Check Switch Section 3 Battery High Function - Ensure voltage above +7V across J14 connector |
| Expected result | Signal at R130 should be high (BAT-LOW-3) |
| Measured | BAT-LOW-3 is logic high |

| Test ID | HT-34 |
| --- | --- |
| Test procedure | Check Switch Section 2 Battery Low Function - Ensure voltage below +6.5V across J14 connector |
| Expected result | Signal at R130 should be low (BAT-LOW-3) |
| Measured | BAT-LOW-3 is logic high |

# Single Switch Section-4

Prerequisites: All power supplies are active, Battery Board or suitable supply is connected

| Test ID | HT-35 |
| --- | --- |
| Test procedure | Check Switch Section 4 ON Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R150 (SECTION4-ON) - Ensure DC voltage at R152 (SECTION4-OFF) |
| Expected result | Signal at R141 should be high (RDS-LOW-4) |
| Measured | RDS-LOW-4 is logic high |
| Comments | SECTION4-ON and SECTION4-OFF signals are reversed at the top level sheet. This needs to be changed |

| Test ID | HT-36 |
| --- | --- |
| Test procedure | Check Switch Section 4 OFF Function - Ensure PWM input of minimum 2kHz with 50% duty cycle at R152 (SECTION4-OFF) - Ensure DC voltage at R150 (SECTION4-ON) |
| Expected result | Signal at R141 should be low (RDS-LOW-4) |
| Measured | RDS-LOW-4 is logic low |
| Comments | SECTION4-ON and SECTION4-OFF signals are reversed at the top level sheet. This needs to be changed |

| Test ID | HT-37 |
| --- | --- |
| Test procedure | Check Switch Section 4 Battery Low Function - Ensure voltage below +6.5V across J15 connector |
| Expected result | Signal at R147 should be low (BAT-LOW-4) |
| Measured | BAT-LOW-4 is logic high |

| Test ID | HT-38 |
| --- | --- |
| Test procedure | Check Switch Section 4 Battery Low Function - Ensure voltage below +6.5V across J14 connector |
| Expected result | Signal at R147 should be low (BAT-LOW-4) |
| Measured | BAT-LOW-4 is logic low |

# Connectors

| Test ID | HT-39 |
| --- | --- |
| Test procedure | Check Detection LED - Connect Detection led to J1-5 - Pull led individually high or low |
| Expected result | Led should turn on and off |
| Measured | LED turning on and off according to the input given. ESD protection circuit is missing here. This needs to be added. |

| Test ID | HT-40 |
| --- | --- |
| Test procedure | Check Input-1 and Input-2 for Key Switch Connector - Drive Input 1 and 2 high/low - Check the voltage at the respective lines |
| Expected result | The Input-1 and Input-2 should reflect the voltage driven at the connector |
| Measured | Input-1 and Input-2 goes high and low according to the voltage at the connector. Without anything connected, Input-1 and Input-2 are always read as high. ESD protection circuit needs to be added here. |

# Top Level

| Test ID | HT-41 |
| --- | --- |
| Test procedure | Check Switch Section Open - Ensure Single Switch Section 1, 2, 3 and 4 are open - Check the resistance between Rail-A and Rail-B |
| Expected result | The resistance measured should be high, indication of an open circuit |
| Measured | Resistance measured = 0.887 Mohms |

| Test ID | HT-41 |
| --- | --- |
| Test procedure | Check Switch Section 1 - Ensure Single Switch Section 1 and 3 are closed - Ensure Single Switch Section 2 and 4 are open - Check the resistance between Rail-A and Rail-B |
| Expected result | The resistance measured should be very less, indication of a short |
| Measured | Resistance Measured = 0.1 ohms |

| Test ID | HT-42 |
| --- | --- |
| Test procedure | Check Switch Section 2 - Ensure Single Switch Section 2 and 4 are closed - Ensure Single Switch Section 1 and 3 are open - Check the resistance between Rail-A and Rail-B |
| Expected result | The resistance measured should be very less, indication of a short |
| Measured | Resistance Measured = 0.1 ohms |

# Required Equipment

| Component | Availability | Comments |
| --- | --- | --- |
| Power Supply Unit – 0 to 12 V | Yes |  |
| Oscilloscope with capture capability | Yes |  |
| UART cables - 2 | Yes | Extra ordered from Farnell |
| SWD programming | Yes |  |
| CAN debugger | Yes | Logic Analyzer can be used as CAN debugger |
| LEDs | Yes |  |
| Cables for connectors | Yes | Requested from Dual Inventive |

# Document Revisions

| Revision | Date | Author | Changes |
| --- | --- | --- | --- |
| 0 | 27-02-2017 | CS | First draft |
| 0.1 | 28-02-2017 | CS | Internal Review Changes – Added 6.4 V section for power supply test, added top level tests and input-1/2 test for J1 connector |
| 0.2 | 15-06-2017 | CS | All test results documented |

# Document Authors

| Initials | Name | E-mail address |
| --- | --- | --- |
| CS | Chethan Shettar | c.shettar@meuleman.io |

[End-of-document]