# Schematic review switch board

Date: 21 December 2016

Attendees: Jerry, San, Gijs

Action list

- Meuleman Electronics will come with a specification proposal for the new Automatic Functional Test (AFT) board.  This specification can be derived from the current tests carried out by the old AFT board.

Notes from conversations over email and phone

Switch board:

- RS232 interface is no longer required, UART is still useful for debugging

- One wire interface is no longer needed

- I2C interface is no longer needed

- Capacitor of 100 nF should be placed across R46, R83, R120 and R157

- Emergency signal generator is no longer required. Only connector (e.g. CN15) remains for mechanical stability. Relevant parts of the switch sections will be updated accordingly

Notes review

The switch board is still in active development. Some suggestions for improvement:

Sheet SwitchPower

- R5, R6 and R7 should be changed to 1206 package so it can handle a higher voltage

Sheet SineWaveGenerator

- Check the enable logic level for U1. Is 3.3V sufficient to enable it?

Sheet SingleSwitchSection

- The microcontroller will be connected to the 3.3V. This is different from the earlier designs. This means that any 5V logic level should be level translated. It is decided to do this instead of changing the analog circuitry. The implementation can be done with an NFET and pull-up resistor

Sheet MeasurementController

- Change C4 and C3 to 22 pF

- Add a diode parallel to R1 (reset circuitry). This enables the capacitor to discharge quickly after the power is removed. This ensures that the processor is reset during a small power interruption

- The power supply of the measurement and drive microcontroller should be made switchable by the control microcontroller

Sheet 11 LedIndicators

- Connector J18 is for future connections, while connector J17 is there for backward compatibility. This could require that the series resistor of J17 is different from J18. Therefore it is better to add a separate filter and series resistor to J17 (68 ohm) and change the series resistor R93, R96, R99 and R102 to 0 ohm